# 🔵 BEARING ENDPOINTS

(These calculate pile bearing capacity)

## 1️⃣ POST /bearing/multiple-cpts/results

### 👉 What it does (simple)

This endpoint:

✔ Takes soil data (CPTs)
✔ Takes pile properties
✔ Takes loads
✔ Calculates pile bearing capacity
✔ Returns a task ID

It does NOT immediately return the result.
It starts a background calculation.

### 🧱 What is "bearing capacity"?

It answers this question:

How much weight can this pile safely carry?

It calculates:

- Shaft resistance (side friction)

- Tip resistance (bottom)

- Negative skin friction

- Characteristic value

- Design value (with safety factors)

### 📊 What results does it create?

When finished, it produces:

- summary_table
→ Final characteristic & design values per pile depth

- all_results
→ All intermediate values per CPT and per depth

You must call /get-task-result to retrieve them.

### 🧠 When do you use this?

When you want to:

"Calculate how much load my pile can carry using multiple CPTs."

## 2️⃣ POST /bearing/multiple-cpts/report

### 👉 What it does

Same calculation as above
BUT it generates a PDF report

The report contains:

- Pile geometry

- Soil data

- Safety factors

- CPT info

- Results tables

- Capacity plots

### 🧠 When do you use this?

When you need:

A professional report for a client or municipality.

# 🟣 GROUPER ENDPOINTS

(These group CPTs statistically)

In Dutch pile design (like NEN9997), you often must:

Combine CPTs into statistically valid groups.

Because:

- Soil varies

- You need statistical reduction (ξ factors)

## 3️⃣ POST /grouper/group-cpts

### 👉 What it does

It automatically:

✔ Looks at calculated bearing capacities
✔ Looks at CPT coordinates
✔ Uses a clustering algorithm
✔ Creates valid CPT subgroups

### 📏 What makes a group valid?

- Variation coefficient < 12%

- Good spatial orientation

- Optional spacing checks

### 🧠 In simple words:

It answers:

"Which CPTs are similar enough to be used together?"

## 4️⃣ POST /grouper/group-metrics

### 👉 What it does

You give it predefined subgroups.

It calculates:

- Variation coefficient

- ξ factors

- Validity checks

- Final group bearing capacity

### 🧠 In simple words:

It answers:

"Is this CPT group statistically valid?"

## 5️⃣ POST /grouper/generate-grouper-report

### 👉 What it does

Generates a PDF report for the grouping results.

Includes:

- Safety factors

- Subgroup tables

- Statistical checks

- Design capacities

# 🟢 RESULT ENDPOINTS

(Task management)

All heavy calculations run in background (Celery task system).

So you must:

- Start calculation

- Receive task ID

- Check status

- Download result

## 6️⃣ GET /get-task-status

### 👉 What it does

Checks if task is:

- PENDING

- RUNNING

- SUCCESS

- FAILED

### 🧠 In simple words:

It answers:

"Is my calculation finished?"

## 7️⃣ GET /get-task-result

### 👉 What it does

Downloads the finished result.

Depending on the task, it returns:

- JSON (tables)

- Or PDF report

# 🧠 COMPLETE WORKFLOW (Very Important)

### Step 1 — Calculate bearing

Call:

POST /bearing/multiple-cpts/results

→ Get task ID

### Step 2 — Wait

Call:

GET /get-task-status?id=...

### Step 3 — Download result

Call:

GET /get-task-result?id=...

### Step 4 — Group CPTs (optional)

Call:

POST /grouper/group-cpts

### Step 5 — Validate groups

Call:

POST /grouper/group-metrics

### Step 6 — Generate group report

Call:

POST /grouper/generate-grouper-report

# 🏗 What This Entire API System Does

It automates:

- CPT interpretation

- Pile bearing capacity calculation

- Statistical reduction (ξ factors)

- Spatial validation

- Safety factor application

- Report generation

Basically:

It replaces manual pile design spreadsheets.